3D Printers

| Makerbot Replicator 2 (FDM) - Build Volume (LxWxH): 285mm x 153mm x 155 mm (11.2” x 6” x 6.1”) - Layer Resolution: 0.1mm / 0.2mm / 0.3mm (0.004”/0.008”/0.012”) - Model Material: PLA - No Support Material (Model material can be used as support but will require manual removal and cleanup afterwards) |  |
| --- | --- |
| Stratasys Mojo (FDM) - Build Volume (LxWxH): 127mm x 127mm x 127 mm (5” x 5” x 5”) - Layer Resolution: 0.178mm (0.007”) - Model Material: ABSplus - Support Material: SR-30 (Soluble in Sodium Hydroxide solution) |  |
| Stratasys uPrint SE plus (FDM) - Build Volume (LxWxH): 203mm x 203mm x 152 mm (8” x 8” x 8”) - Layer Resolution: 0.254mm / 0.33mm (0.01” / 0.013”) - Model Material: ABSplus - Support Material: SR-30 (Soluble in Sodium Hydroxide solution) |  |
| Stratasys Dimension SST 1200es (FDM) - Build Volume (LxWxH): 254mm x 254mm x 305 mm (10” x 10” x 12”) - Layer Resolution: 0.254mm / 0.33mm (0.01” / 0.013”) - Model Material: ABSplus - Support Material: P400SR (Soluble in Sodium Hydroxide solution) |  |
| Objet 350 Connex 3 (Polyjet) - Build Volume (LxWxH): 340mm x 340mm x 200mm (13.4” x 13.4” x 7.9”) - Layer Resolution: 0.016mm or 0.03mm (0.0006” or 0.001”) - Model Material: Multiple material options + digital Materials - Support Material: SUP705 gel like photopolymer (removed using waterjet) |  |